Nama : Adi Saepul Anwar

NPM : 43A87006200129

Matkul : Jarkom 2

Kelas : S1/TI/4/AM

Topologi

Konfigurasi

Router 0

interface FastEthernet0/0

ip address 10.10.10.30 255.255.255.248

duplex auto

speed auto

!

interface FastEthernet0/1

ip address 20.20.20.30 255.255.255.248

duplex auto

speed auto

!

interface Serial0/0/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router eigrp 200

network 10.10.10.0 0.0.0.255

network 20.20.20.0 0.0.0.127

network 10.10.10.24 0.0.0.7

network 20.20.20.24 0.0.0.7

auto-summary

Router 1

interface FastEthernet0/0

ip address 30.30.30.30 255.255.255.248

duplex auto

speed auto

!

interface FastEthernet0/1

ip address 20.20.20.29 255.255.255.248

duplex auto

speed auto

!

interface Serial0/0/0

ip address 40.40.40.30 255.255.255.248

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router eigrp 200

network 20.20.20.0 0.0.0.127

network 30.30.30.0 0.0.0.255

network 40.40.40.0 0.0.0.255

network 20.20.20.24 0.0.0.7

network 30.30.30.24 0.0.0.7

network 40.40.40.24 0.0.0.7

auto-summary

Router 2

interface FastEthernet0/0

ip address 50.50.50.30 255.255.255.248

duplex auto

speed auto

!

interface FastEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

no ip address

clock rate 2000000

shutdown

!

interface Serial0/0/1

ip address 40.40.40.29 255.255.255.248

clock rate 2000000

!

interface Vlan1

no ip address

shutdown

!

router eigrp 200

network 50.50.50.24 0.0.0.7

network 40.40.40.24 0.0.0.7

auto-summary

Table Routing

| Router 0 | Router 1 | Router 2 |
| --- | --- | --- |
| Gateway of last resort is not set 10.0.0.0/8 is variably subnetted, 2 subnets, 2 masks D 10.0.0.0/8 is a summary, 00:01:21, Null0 C 10.10.10.24/29 is directly connected, FastEthernet0/0 20.0.0.0/8 is variably subnetted, 2 subnets, 2 masks D 20.0.0.0/8 is a summary, 00:01:21, Null0 C 20.20.20.24/29 is directly connected, FastEthernet0/1 D 30.0.0.0/8 [90/30720] via 20.20.20.29, 00:03:19, FastEthernet0/1 D 40.0.0.0/8 [90/2172416] via 20.20.20.29, 00:03:14, FastEthernet0/1 D 50.0.0.0/8 [90/2174976] via 20.20.20.29, 00:03:14, FastEthernet0/1 | Gateway of last resort is not set D 10.0.0.0/8 [90/30720] via 20.20.20.30, 00:13:27, FastEthernet0/1 20.0.0.0/8 is variably subnetted, 2 subnets, 2 masks D 20.0.0.0/8 is a summary, 00:32:00, Null0 C 20.20.20.24/29 is directly connected, FastEthernet0/1 30.0.0.0/8 is variably subnetted, 2 subnets, 2 masks D 30.0.0.0/8 is a summary, 00:32:00, Null0 C 30.30.30.24/29 is directly connected, FastEthernet0/0 40.0.0.0/8 is variably subnetted, 2 subnets, 2 masks D 40.0.0.0/8 is a summary, 00:30:40, Null0 C 40.40.40.24/29 is directly connected, Serial0/0/0 D 50.0.0.0/8 [90/2172416] via 40.40.40.29, 00:09:13, Serial0/0/0 | Gateway of last resort is not set D 10.0.0.0/8 [90/2174976] via 40.40.40.30, 00:08:46, Serial0/0/1 D 20.0.0.0/8 [90/2172416] via 40.40.40.30, 00:08:46, Serial0/0/1 D 30.0.0.0/8 [90/2172416] via 40.40.40.30, 00:08:46, Serial0/0/1 40.0.0.0/8 is variably subnetted, 2 subnets, 2 masks D 40.0.0.0/8 is a summary, 00:08:46, Null0 C 40.40.40.24/29 is directly connected, Serial0/0/1 50.0.0.0/8 is variably subnetted, 2 subnets, 2 masks D 50.0.0.0/8 is a summary, 00:08:46, Null0 C 50.50.50.24/29 is directly connected, FastEthernet0/0 |